Atari USB2PC Build Instructions for AspeQt(13L)

Building an Atari USB2PC cable is very simple and parts can be purchased on Amazon for under $15.00!   Below outlines the pin layouts for building two different cables using a PL2303TA chipset and/or a FT232RL chipset.   Both cables can be used with the RespeQt and the AspeQt disk drive/peripheral device emulator. The FT232RL chipset version can also be used with the APE Atarimax software.

As always, our BBS and more info can be found at:   https://13leader.net

SIO2PC Cable Using USB-TTL PL2303TA Chipset

RespeQt Handshake = “None”

SIO Pins						USB-TTL PL2303TA Chipset

1 – Clock Input

2- Clock Output

3- Data In		 				TXD

4- Ground		    				GND

5-Data Out		   				RXD

6- Ground

7-Command Signal

8- Motor

9-Proceed

10- +5 Volts

11- Audio n

12- +12 Volts

13- Interrupt

SIO2PC Cable Using USB-TTL FTDI FT232RL Chipset

RespeQt Handshake = “CTS”

SIO Pins						USB-TTL FTDI FT232RL Chipset

1 – Clock Input

2- Clock Output

3- Data In		 				TXD

4- Ground 			GND

5-Data Out	 		RXD

6- Ground

7-Command Signal         		CTS

8- Motor

9-Proceed

10- +5 Volts

11- Audio n

12- +12 Volts

13- Interrupt